| Indian Academy of Sciences, Bengaluru Indian National Science Academy, New Delhi The National Academy of Sciences India, Pryagraj | SCIENCE ACADEMIES’ SUMMER RESEARCH FELLOWSHIP PROGRAMME P. B. No. 8005, C. V. Raman Avenue, Sadashivanagar Psot, Bengaluru 560 080 Telephone: (080) 2266 1207, 2266 1202, 2266 1221, Fax: (080) 2361 6094 Email: sumfel@ias.ac.in Website: www.ias.ac.in |
| --- | --- |

| Title | Detection of Convection-Triggered Gravity Waves in the Atmospheric Boundary Layer Using LiDAR Observations |
| --- | --- |
| Name | Mr. Sudhan R |
| Application Number | ENGS1672 |
| Candidate’s Institute | Sri Eshwar College of Engineering, Affiliated to Anna University |
| Name of the Guide | Dr. Y. Bhavani Kumar |
| Name of the Institute | National Atmospheric Research Laboratory (NARL) |
| Report | 8 – Week Report |

### 1. WORK CARRIED OUT AFTER THE FIRST FOUR WEEKS

After completing the initial understanding of LiDAR data structure and preliminary range-time visualization, the second phase of the project focused on deeper signal extraction, frequency analysis, and interpretation.

- The dataset provided contained over 850 LiDAR profiles in Licel .C14 binary format, each corresponding to a 30-second vertical scan.

- All files were read using Python and converted into a structured format. A NumPy 2D array was generated where rows represented vertical bins (altitude) and columns represented time.

- The total vertical resolution was ~7.5 m per bin, and the data extended up to ~3.1 km altitude.

Figure 1: RTI - Range Time Intensity

2. SELECTION OF ALTITUDES AND TIME SERIES ANALYSIS

Based on RTI visual inspection, altitude bins between 750 m and 2600 m showed clear periodic signal variations. Ten specific bins were selected at 200–250 m intervals for detailed time-series analysis.

- Time-series plots were generated for each altitude to observe visual oscillations.

- These plots helped in identifying potential regions where convection-triggered gravity wave activity was visible.

### 3. SPECTRAL (FFT) ANALYSIS

Fast Fourier Transform (FFT) was applied to each time series to detect dominant frequencies.

- Dominant peaks were identified in the range of 0.00027 to 0.00043 Hz, corresponding to wave periods of 38 to 60 minutes.

- These values match theoretical expectations for gravity waves generated by convective activity in the ABL.

Figure 2: Fast Fourier Transform (FFT)

### 4.  SIGNAL INTENSITY VS TIME

Figure 3: Signal Intensity vs Time

### 5. INTERPRETATION OF RESULTS

- Dominant gravity wave signatures were observed at ~1762.5 m, 2167.5 m, and 2572.5 m.

- These layers showed both visual periodicity and matching frequency peaks.

- Above 2700 m, the signal dropped significantly, and below 500 m, wave signatures were weak or noisy.

| Bin Index | Altitude (m) | Dominant Frequency (Hz) | Wave Period (min) |
| --- | --- | --- | --- |
| 100 | 750.0 | 0.015044 | 1.10 |
| 127 | 952.5 | 0.015044 | 1.10 |
| 154 | 1155.0 | 0.015044 | 1.10 |
| 181 | 1357.5 | 0.000040 | 421.0 |
| 208 | 1560.0 | 0.000079 | 210.5 |
| 235 | 1762.5 | 0.000435 | 38.2 |
| 262 | 1965.0 | 0.000040 | 421.0 |
| 289 | 2167.5 | 0.000277 | 60.1 |
| 316 | 2370.0 | 0.000040 | 421.0 |
| 343 | 2572.5 | 0.000277 | 60.1 |

Table 1: FFT Summary Table

| Altitude (m) | Wave Period (min) | Interpretation |
| --- | --- | --- |
| 750–1155 | ~1.1 mins | Could be high-frequency noise or local turbulence, not gravity waves |
| 1357–2370 | 210–421 mins | This is in the long-period gravity wave or convective plume range |
| 1762.5 | ~38 mins | This is very relevant! Likely a gravity wave signal |
| 2167.5, 2572.5 | ~60 mins | Classic CBL convection-triggered wave periods |

Table 2: Interpretation

### 6. TOOLS AND METHODS USED

- Google Colab (Python): For binary file reading, reshaping, plotting, FFT

- NumPy, Matplotlib, SciPy: For signal processing

- Licel Binary Format Decoding: Using manual specs

- Manual RTI Inspection + Programmatic Filtering

- FFT + Time-Domain Overlay Interpretation

### 7. CONCLUSION

This project successfully demonstrated the use of LiDAR signal profiles to detect convection-triggered gravity waves within the atmospheric boundary layer. The chosen methodology — combining time-series analysis with frequency domain inspection — confirmed the presence of periodic wave patterns in the 1.7 km to 2.6 km altitude band.

8. REFERENCES

[1] Y. B. Kumar, V. B. Rao, and D. Narayana Rao, "Wave structures in the atmospheric boundary layer observed with lidar," Journal of Atmospheric and Solar-Terrestrial Physics, vol. 63, no. 16, pp. 1641–1650, 2001.

[2] T. Foken, "Micrometeorology," Springer Atmospheric Sciences, Berlin, Heidelberg: Springer, 2008.

[3] Licel GmbH, “Transient Recorder Data Format,” Licel Instruments, Berlin, Germany. [Online]. Available: https://www.licel.com/manuals/programmingManual.pdf

[4] M. J. Alexander et al., "Recent developments in gravity-wave effects in climate models and the global distribution of gravity-wave momentum flux from observations and models," Quarterly Journal of the Royal Meteorological Society, vol. 136, no. 650, pp. 1103–1124, Apr. 2010.

[5] S. Mallik, P. Kumar, and R. Singh, "Atmospheric gravity waves and their role in the middle atmosphere," Current Science, vol. 111, no. 3, pp. 495–503, 2016.

[6] J. W. Goodman, Statistical Optics, 2nd ed., Wiley, 2015.

[7] E. E. Clothiaux and M. L. Van Baelen, "Technical Note: Use of lidar for boundary layer height estimation," Atmospheric Measurement Techniques, vol. 6, no. 12, pp. 3253–3264, 2013.

[8] J. D. Hunter, "Matplotlib: A 2D graphics environment," Computing in Science & Engineering, vol. 9, no. 3, pp. 90–95, 2007.

[9] W. McKinney, "Data structures for statistical computing in Python," in Proc. 9th Python in Science Conf., Austin, TX, USA, 2010, pp. 51–56.